K1 SSB mod. performance test.

20 m measurements:

USB mode, 1KHz tone to input mike, output 5 watts

Higher spurious:	-50dBc @ 11 MHz

II armonic:		-53 dBc

III armonic:		-63 dBc

Carrier suppression:	-36 dBc

Power  @ freq.:

| KHz | SSB Watts | CW Watts |
| --- | --- | --- |
| 14.000 | 4.6 | 3 |
| 14.050 | 5 | 3.5 |
| 14.075 |  | 3.7 |
| 14.100 | 5.2 | 3.7 |
| 14.150 | 5.3 |  |
| 14.200 | 5.2 |  |
| 14.250 | 5.1 |  |

40 m measurements:

LSB mode, 1KHz tone to input mike, output 5 W

Higher spurious:	-48dBc @ 4MHz

II armonic:		-51dBc

III armonic:		-58 dBc

Carrier suppression:	-36 dBc

Power @ freq.:

| KHz | SSB Watts | CW Watts |
| --- | --- | --- |
| 7000 | 4.9 | 4.2 |
| 7050 | 4.9 | 4.5 |
| 7100 | 4.8 |  |
| 7150 | 4.6 |  |
| 7200 | 4.4 |  |
| 7250 | 4.2 |  |

I am not able to measure SSB side-band suppression because my Analyzer (Advantest TR4131) has a minimum Freq. Span of 100 KHz.

The SSB side suppression can be evaluated from SSB Filter shape in the following plot.

RX filter shape:

This filter is made with 6 x 4915 KHz crystal.

BFO Freq. Positions are:

20 m USB: 4916.8 KHz

40m LSB :4913.8 KHz

TX filter shape:

N

This filter uses  4  x  4915 KHz Crystal

BFO position are the same as RX

On 20 m the side band supression can be estimated  > 35 dB

On 40 m > 25 dB

NB: better results are obtained if the TX filter is equal to  RX filter.

The filters plot are  made with my network project:

Relevant informations to:

http://it.geocities.com/giulianoi0cg/digital_power_meter.html

For informations write to:

Giuliano I0CG

giulianori@libero.it

web page:

http://it.geocities.com/giulianoi0cg