K1 RX SSB Modification

Here has explained the RX modification first; you can test it, then, you  increment modifications  to add TX modification too.

PCB protype board

Assembling RX part

#### RF PCB preliminary mod.setup

On the K1 RF board the following components are removed:

R21, C38, RFC1

R21 must be   removed to obtain  a  flat audio response on SSB. If R21 is not removed the audio response has optimized  for CW (frequencyes higher then 1 KHz are cut off).

Added components:

Insert 4 female pins in place of C38 and RFC1. (see RF PCB Photo in the following)

Insert a female pin to have the XFILT/TONE signal available. This signal is on the PCB vias near the RF-P1 connector.

Insert Noise blanker connector  (J1) ( 8 female pins connector)

To extend the K1 frequency range insert an NPO cap.  68 pF   parallel to C2 (120 pF).

With this last mod. the frequency range is about 250 KHz (7-7250 and 14- 14250 KHz on my dual band K1)

This component change value:

C38 become 100 pF

To restore the normal K1 functionality you can fit C38 and RFC1 again on the female pins inserted on RF PCB.

# RX board assembling

Install all components of the RX section (see schematic and top layout). The SMD components in the bottom board are preasembled.

When ready, the SSB  mod. board, is  plugged  in the Noise-blanker connector and the others added pin. (no wires are necessary)

# RX Hardware SETUP

Set V1 to 4.9 Vdc (pin 2 of U1)

Set V2  to 1.1 Vdc (Pin 6 of U1)

Set filter 3 to 200 Hz (FL3) then adjust C12 for 4916.500 KHz (+/-100 Hz)  for USB mod. (frequency test point on pin 6 of  U3)

Set filter 1 to 850 Hz (FL1) verify 4913.500 KHz (+/-100Hz)  for LSB  mod. (frequency test point  on pin 6 of  U3)

With filter 2 ( FL2)   adjust C34 for better CW note (about 4915.7 KHz)

Set filter 2 in the range 250 - 800 Hz to select the CW filter bandwidth preferred

Verify that relay has switched “on” when FL1 and FL3 are selected.

Operation:

To set LSB mode press WPM+ and select FL1 (SSB filter and LSB frequency are selected)

To set USB mode press WPM+ and select FL3  (SSB filter and USB frequency are selected)

To set CW operation selected FL2

All 3 CW filters are availablle if are selected in the 250-800 Hz range. In this case SSB filter are not activated.

Assembling TX part.

K1 RF PCB modification

Removed components:

- Remove X6

- RFC2 must be shorted

Added components:

- Add two female pins inside X6 quartz (to restore mod. X6 can be fitted on these pins)

- Add one female pins to have a 8V supply on the mod. board. (this supply voltage is available on a PCB via on front C18 K1 capacitor)

- same components can be inserted into  RF board to change into Ab-class the final TX amplifier. (instead of class-C). This  mod. is necessary to enanche the SSB quality.To add this mod. follows these steps:

- remove R31 and R32 from the GND connection (do not remove the Q7 base connection)

- add a standard diode (i.e.1N4148 ) in series to R31 , R32 .(catode to GND and anode to R31/32)

- polarize the diode via the 6T voltage (available on the R38/R28 terminals) through an 1K ohm resistor.

- Open terminal 4 of T3 from GND and connect it to the anode of diode.

- bypass the diode with a 0.1 capacitor

For more clarity see the following schematic of the performed mod. :

Class-AB mod

TX board assembling:

See the overall TX and RX mod. schematics and relevant bill of materials.

As RX part only the top board components must be assembled. (surface components are pre-assembled)

The board is  plugged-in  on the Noise-blanker  connector and the others added pin.

A shield cable (2” length) must be connected from 2 pin connector P6 and the pin added on X6 place

TX  Hardware SETUP

Select K1 filter 1 or 3 then  go in TX mode ( see below) then adjust V4 Pot. for maximum  carrier suppression (you can use a Scope or another receiver)

Insert mike then adjust V3 pot. for suitable mike sensitivity.

OPERATION:

Reception:

To set LSB mode press WPM+ and select FL1 (SSB filter and LSB frequency are selected)

To set USB mode press WPM+ and select FL3  (SSB filter and USB frequency are selected)

To set CW operation selected FL2

All 3 CW filters are availablle if are selected in the 250-800 Hz range. In this case SSB filter aren’t activated.

Transmission:

Transmission is performed using the TUNE Mode: press the WPM+ and WPM- simultaneously.

To return on RX press any key.

Schematic of RX and TX SSB modification.

The red points  shows female pins added on the  K1 RF PCB

Top layout

Bottom layout . The SMD components are pre-assembled

On this side install only the following components:

8 pin connector P2

2 pin connector K1/c30

1 pin connector XFIL

1 pin connector P7_1

1 pin connector P7_2

1 pin connector +8V

Bill of materials

NAME         VALUE

C1             100PF

C2             100PF

C3             1kpF

C4             33PF

C5             47PF

C6             47PF

C7             47PF

C8             33PF

C9             220pF

C10            33pF

C11            100PF

C12            1/10PF TRIM.

C13            68pF

C14            2nF

C15            10nF

C16            2.2UF

C17            2.2UF

C18            1UF

C19            2.2UF         15V

C20            100PF

C21            100pF

C22            22nF

C23            10nF

C24            33PF

C25            47PF

C26            47PF

C27            4.7nF

C28            10nF

C29            2.2UF

C30            1uF           15V

C31            470pF

C32            47pF          15V

C33            33PF          15V

C34            1/10pF

D1             1N4148

D2             1N4148

D3             1N4148

D4             1N4148

J1             NC/3-Ckt

L1             100uH

P1               2-pin

P2               8-pin

P6               2-pin

P7               2-pin

Q1               BC318

Q2               2N3819

Q3               BC318

Q4               BC318

Q5               BC318

Q6               2N1131

Q7               BC318

Q8               BC318

R1               0 27k OHM

R2               27k OHM

R3               1500 OHM

R4               2200 OHM

R5               47k OHM

R6               2.2k OHM

R7               330k OHM

R8               2k OHM

R9              47k OHM

R10              15k OHM

R11              15k OHM

R12             2.2k OHM

R13              2.2k OHM

R14              15k OHM

R15              2.2k OHM

R16              47 OHM

R17              15K OHM

R18              1k OHM

R19              27k OHM

R20              47k OHM

R21              4.7k OHM

R22              10k OHM

R23              10k OHM

R24              27k OHM

R25              47k OHM

R26              2.2K OHM

R27              2.2k OHM

R28              1500 OHM

R29              15K OHM

REL1             TF2-5V NAIS

RFC1

U1a              LM2904

U1b              LM2904

U2a              LM2904

U2b              LM2904

U3               SA602

V1               10k OHM     Pot

V2               10k OHM     Pot

V3               1M  OHM     Pot

V4               1M  OHM     Pot

X1              XTAL          4915KHz

X2              XTAL          4915KHz

X3              XTAL          4915KHz

X4              XTAL          4915KHz

X5              XTAL          4915KHz

X6              XTAL          4915KHz

X7              XTAL          4915KHz

X8              XTAL          4915KHz

X9              XTAL          4915KHz

X10             XTAL          4915KHz

X11             XTAL          4915KHz

X12             XTAL          4915KHz

Z1              Zener             3.6 V